MATLAB ESR Estimator – Working Explanation

1. persistent ESR_filt

A persistent variable is created to store the previous ESR value between function calls. This allows the implementation of a recursive digital low-pass filter and prevents the value from resetting at every simulation step.

2. if isempty(ESR_filt)

ESR_filt = 0;

end

During the first execution of the function, ESR_filt does not exist. Therefore, it is initialized to zero to provide a valid starting value for the filter.

3. I_CAP_STEP = 0.3;

This represents the capacitor current ripple step (ΔIC) used in ESR calculation.

ΔIC = 0.3 A

This value is obtained from the converter analysis and is assumed constant for the estimation process.

4. K = 10;

K is a calibration factor used to compensate for signal scaling introduced by the amplifier stages, measurement circuitry, and practical implementation errors.

5. ESR estimation is based on the capacitor ESR voltage relationship:

VESR = ESR × ΔIC

where:

VESR = ESR-induced ripple voltage

ESR = Equivalent Series Resistance

ΔIC = Capacitor current ripple

6. Rearranging the above equation gives

ESR = VESR / ΔIC

This is the fundamental formula used for ESR estimation.

7. ESR_inst = (K * abs(V_adc)) / (Gain_Amp * I_CAP_STEP);

This line calculates the instantaneous ESR value.

Since the measured voltage has already been amplified:

VESR = Vadc / GainAmp

Substituting into the ESR equation:

ESRinst = (K × |Vadc|) / (GainAmp × ΔIC)

where:

Vadc = ETS ripple voltage

GainAmp = Total amplifier gain

ΔIC = Capacitor current ripple

K = Calibration factor

The abs() function is used because ESR is always positive regardless of ripple polarity.

8. alpha = 0.05;

alpha is the digital filter coefficient.

α = 0.05

A small alpha value provides:

• Strong noise reduction

• Smooth ESR output

• Slower response to sudden changes

9. ESR_filt = alpha * ESR_inst + (1 - alpha) * ESR_filt;

This implements a first-order Infinite Impulse Response (IIR) low-pass filter.

Mathematically:

ESRfilt(k) = α × ESRinst(k) + (1 − α) × ESRfilt(k−1)

Substituting α = 0.05:

ESRfilt(k) = 0.05 × ESRinst(k) + 0.95 × ESRfilt(k−1)

This equation smooths the ESR estimate by combining:

• 5% of the current ESR estimate

• 95% of the previous ESR estimate

10. ESR = ESR_filt;

The filtered ESR value is assigned to the output variable.

This is the final estimated ESR displayed by the Simulink model and represents the real-time health condition of the output capacitor.

Final Mathematical Equation Implemented by the Code

ESRinst = (K × |Vadc|) / (GainAmp × ΔIC)

ESRfilt(k) = α × ESRinst(k) + (1 − α) × ESRfilt(k−1

Substituting the project values:

K = 10

ΔIC = 0.3 A

α = 0.05

ESRinst = (10 × |Vadc|) / (GainAmp × 0.3)

ESR(k) = 0.05 × [(10 × |Vadc|)/(GainAmp × 0.3)] + 0.95 × ESR(k−1)

This equation forms the complete real-time ESR estimation algorithm used in the project.