# Main Purpose of the ESR Estimation Project

## 1. Project Objective

The primary objective of this project is not to design a boost converter. The boost converter is only used as a platform for testing.

The actual goal of the project is:

To estimate the Equivalent Series Resistance (ESR) of the output capacitor online, without disconnecting the capacitor, without injecting external test signals, and without using expensive laboratory measurement equipment.

# 2. Importance of ESR

A practical capacitor is not ideal. It can be represented as an ideal capacitor in series with a small resistance called Equivalent Series Resistance (ESR).

As capacitors age:

- ESR increases

- Output voltage ripple increases

- Converter efficiency decreases

- Capacitor heating increases

- Reliability decreases

- Converter failure may occur

Typical ESR values:

| Capacitor Condition | ESR |
| --- | --- |
| New Capacitor | 0.05 Ω |
| Healthy Capacitor | 0.1 Ω |
| Aging Capacitor | 0.5 Ω |
| Failed Capacitor | >1 Ω |

The purpose of the paper is to continuously monitor this ESR variation while the converter is operating.

# 3. Limitations of Traditional ESR Measurement

Traditional ESR measurement methods use:

- LCR Meter

- ESR Meter

- Impedance Analyzer

These methods require:

- Converter shutdown

- Capacitor disconnection

- Offline measurement

Therefore they are not suitable for real-time monitoring in practical power electronic systems.

# 4. Proposed Solution in the Paper

Instead of disconnecting the capacitor, the paper estimates ESR from the output voltage ripple already present in the boost converter.

The method follows:

Output Voltage Ripple → Ripple Extraction → Step Detection → ESR Calculation

Thus ESR can be estimated while the converter continues operating normally.

# 5. Principle of Operation

The boost converter operates using periodic switching of the MOSFET.

## MOSFET ON State

- Inductor stores energy.

- Capacitor supplies load current.

- Output voltage decreases slightly.

## MOSFET OFF State

- Inductor releases stored energy.

- Capacitor charges.

- Output voltage increases slightly.

This creates a periodic output voltage ripple.

The ripple waveform contains information about the capacitor ESR.

# 6. ESR Estimation Principle

The paper uses the relationship:

ESR = ΔV_ESR / ΔI_C

Where:

ΔV_ESR = ESR-induced voltage step

ΔI_C = Capacitor current step

By measuring the voltage step produced by the ESR and dividing it by the capacitor current change, the ESR can be estimated.

# 7. Ripple Components

The total output ripple consists of two parts:

Total Ripple = Capacitive Ripple + ESR Ripple

## Capacitive Ripple

- Slow variation

- Due to capacitor charging and discharging

## ESR Ripple

- Fast voltage step

- Appears during switching transitions

The paper extracts only the ESR component because it directly contains ESR information.

# 8. Equivalent Time Sampling (ETS)

The ripple amplitude is extremely small, typically only a few millivolts.

Direct ADC sampling cannot accurately reconstruct the waveform.

Therefore the paper uses Equivalent Time Sampling (ETS).

ETS works by collecting samples from many switching cycles and reconstructing a high-resolution ripple waveform.

Example:

Cycle 1 → Sample Point 1

Cycle 2 → Sample Point 2

Cycle 3 → Sample Point 3

…

Cycle 200 → Sample Point 200

After combining all samples, the complete ripple waveform is reconstructed.

# 9. Purpose of Each Block in the Simulink Model

| Simulink Block | Function |
| --- | --- |
| Boost Converter | Generates practical capacitor ripple |
| Voltage Measurement | Measures output voltage ripple |
| Differential Amplifier | Amplifies small ripple signals |
| Low Pass Filter (LPF) | Removes switching noise |
| Saturation Block | Emulates analog signal protection |
| Zero Order Hold | Models ADC sampling process |
| Unit Delay | Stores previous sample |
| Sum Block | Computes ripple difference |
| Threshold Detector | Detects ESR voltage steps |
| MATLAB Function | Calculates ESR |
| Scope Blocks | Displays waveforms |

# 10. Operation of the Complete System

The complete measurement chain is:

Boost Converter → Output Voltage Ripple → Voltage Measurement → Amplification → Low Pass Filtering → ADC Sampling → Ripple Extraction → Step Detection → ESR Calculation

This sequence reproduces the complete ESR estimation methodology proposed in the paper.

# 11. Expected Experimental Validation

The ideal validation process is:

## Experiment 1

Actual Capacitor ESR = 0.1 Ω

Estimated ESR ≈ 0.1 Ω

## Experiment 2

Actual Capacitor ESR = 0.5 Ω

Estimated ESR ≈ 0.5 Ω

## Experiment 3

Actual Capacitor ESR = 1 Ω

Estimated ESR ≈ 1 Ω

If the estimated ESR follows the actual ESR, the method successfully detects capacitor degradation.

# 12. Final Project Statement

The project develops an online capacitor health monitoring system for a boost converter by estimating the output capacitor ESR from output voltage ripple using ripple extraction and equivalent time sampling techniques without disconnecting the capacitor from the converter.